Task-3

Problem-1:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define PIR_PIN D5      // PIR output

// Variables to handle motion

bool motionState = false;     // current motion state

bool lastMotionState = false; // previous motion state

unsigned long motionTime = 0;

const unsigned long motionDelay = 2000; // 2 seconds debounce

void setup() {

pinMode(PIR_PIN, INPUT);

// Initialize OLED

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

while(true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(0, 20);

display.println("System Ready");

display.display();

delay(2000);  // PIR warm-up time

}

void loop() {

int pirValue = digitalRead(PIR_PIN);

// Debounce / filter motion detection

if (pirValue == HIGH) {

motionState = true;

motionTime = millis(); // record time of last motion

} else {

// if no motion for more than delay, reset state

if (millis() - motionTime > motionDelay) {

motionState = false;

}

}

// Update OLED only if state changes

if (motionState != lastMotionState) {

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0,0);

display.println("PIR Motion Status");

display.setTextSize(2);

display.setCursor(0,25);

if (motionState) {

display.println("MOTION");

} else {

display.println("NO MOTION");

}

display.display();

lastMotionState = motionState;

}

delay(100); // small delay

}

Problem-2:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define IR_PIN D5   // IR sensor output

bool objectDetected = false;      // current stable state

bool lastState = false;           // last stable state

unsigned long lastChange = 0;

const unsigned long debounceTime = 200;  // 200 ms debounce

void setup() {

pinMode(IR_PIN, INPUT);

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)){

while(true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(0, 20);

display.println("IR Sensor Ready");

display.display();

delay(2000);

}

void loop() {

int sensorValue = digitalRead(IR_PIN);

bool currentState = (sensorValue == LOW);  // LOW = object detected

// Check if state changed

if (currentState != lastState) {

lastChange = millis();  // reset debounce timer

}

// Only update state if stable for debounceTime

if ((millis() - lastChange) > debounceTime) {

objectDetected = currentState;

}

lastState = currentState;

// Update OLED

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0,0);

display.println("IR Object Detection");

display.setTextSize(2);

display.setCursor(0,25);

if (objectDetected) {

display.println("OBJECT");

} else {

display.println("NO OBJECT");

}

display.display();

delay(50);  // small delay

}

Problem-3:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define FLAME_PIN D7   // Flame sensor digital output

// Use true if your sensor is active-low (LOW = flame detected)

bool activeLow = true;

bool flameDetected = false;

bool lastState = false;

unsigned long lastChange = 0;

const unsigned long debounceTime = 200; // 200 ms debounce

void setup() {

pinMode(FLAME_PIN, INPUT);

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)){

while(true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(0, 20);

display.println("Flame Sensor Ready");

display.display();

delay(2000);

}

void loop() {

int sensorValue = digitalRead(FLAME_PIN);

// Determine flame detection based on sensor logic

bool currentState = activeLow ? (sensorValue == LOW) : (sensorValue == HIGH);

// Debounce: only change if stable for 200 ms

if (currentState != lastState) {

lastChange = millis();

}

if (millis() - lastChange > debounceTime) {

flameDetected = currentState;

}

lastState = currentState;

// Update OLED display

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0,0);

display.println("Fire Detection Status");

display.setTextSize(2);

display.setCursor(0,25);

if (flameDetected) {

display.println("FIRE!");

} else {

display.println("SAFE");

}

display.display();

delay(100);

}

Problem-4:

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define MQ2_A0_PIN A0   // Analog pin

#define MQ2_D0_PIN D5   // Digital pin (threshold)

const int ANALOG_THRESHOLD = 600;  // adjust based on environment

void setup() {

pinMode(MQ2_D0_PIN, INPUT);

pinMode(MQ2_A0_PIN, INPUT);

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)){

while(true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(0,20);

display.println("MQ2 Ready");

display.display();

delay(2000);

}

void loop() {

int analogValue = analogRead(MQ2_A0_PIN);   // 0-1023

int digitalValue = digitalRead(MQ2_D0_PIN); // HIGH/LOW

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0,0);

display.println("MQ2 Smoke Detection");

display.setTextSize(2);

display.setCursor(0,25);

// Check both A0 and D0

if( analogValue > ANALOG_THRESHOLD){

display.println("SMOKE!");

} else {

display.println("SAFE");

}

display.setTextSize(1);

display.setCursor(0,58);

display.print("Analog: "); display.print(analogValue);

display.print(" D0: "); display.println(digitalValue);

display.display();

delay(200);

}

Problem-8: